AssemBLY & C - Programming With AT89S52 (8051)

Workshop Manual

TEAM FALCON

NAME:

SEMESTER:

Contents

| Topic | Page |
| --- | --- |
| Proteus | 2 |
| Keil | 6 |
| Intel-8051 | 9 |
| 8051 I/O Programming | 11 |
| Assembly Programming | 12 |
| C Programming | 15 |
| Timers | 18 |
| Mini-Project | 23 |

Proteus

Proteus Design Suite, is an EDA tool used by professionals and students for Schematic development and PCB design.

The major advantage of this simulator is it provides a variety of MCU for simulation via coding. This feature reduces testing time on the hardware design.

For further information visit https://www.labcenter.com/

Offers coding environment in Assemble, C/C++, Energia ide, Arduino ide and many more.

Supports many Processors from Atmel, Ti, and Intel.

In the upcoming Assembly coding and Embedded C coding of 8051, we extensively use Proteus Design Suite professional edition 8. This helps you to render the simulation of circuits in short time. Make alterations and the final working code can be incorporated in the 8051.

Working with Proteus

- Creating a new Project

- Enter the Name and Project file path.

- Selection of Template size, (Default size can be picked).

- Based on preference PCB firmware template can be chosen.

- For 8051, select create a Firmware project.

Family: 8051.

Controller: AT89C52 (Similar version of AT89S52, board provided).

Complier: ASEM-51.

- Summary is given, Click Finish.

- Schematic and Source code appears on the screen.

- Read program from External file, or enter the code in the Source code tab.

- Make the circuit connections and run the code.

KEIL

Keil is an embedded programming tool. This is used to code compliers, Macro assemblers, Emulators, Debuggers, Programming ARM Cortex, Embedded C programing, and Creation of INTEL HEX files for microcontrollers and microprocessors.

Proteus is used to simulate the design and for testing. The tested code is then written in KEIL for Embedded C (C51 version of Keil µVision50). The code is converted to Intel Hex file format to load it into the microcontroller.

Working with KEIL

- Open a New Project / New µVision project.

- Search for the device to be programmed. (AT89S52).

- Got to Target, Options for Target.

- Make the crystal oscillator frequency into 11.0592 MHz, and in the Output Tab, select the HEX file output (HEX-80).

- Under Target go to source group and add new file (C or ASM file).

- Program and Build the code using F7.

- The HEX file will be in the target folder, inside Objects folder.

SPI Programming With Arduino to 8051

Program the Arduino to read the hex file, and send it serially to 8051

89S52Programmer.exe is used for this purpose to detect, erase, and upload the hex file.

8051- Intel

(AT89S52-Atmel, used in the text)

Architecture

- Clock for the Processor can be given from 4Mhz to 40Mhz range.

- There are two 16 bit timers: Timer0 and Timer1 in 8051 (The board used here AT89S52 has three Timer0, Timer1, and Timer2

- On chip 8KB of ROM (ISP programmable: Programming via Arduino, also known as In System Programming with SPI derail interfacing. Using MOSI, MISO, SCLK.) , and 256B of RAM.

- Full Duplex URAT.

- For a more detailed description visit   http://ww1.microchip.com/downloads/en/DeviceDoc/doc1919.pdf

Registers of 8051

- Most widely used are A (Accumulator), B, and R0 to R7. Other important register is PC (Program Counter, to point to the current address in the program from memory).

- For mathematical applications Status Register is used.

Simulation of Assembly Programs

- Can be done via Proteus. There are other options like ProView32, Edsim51, and MCU 8051.

- Edsim51 requires Java Runtime Environment (JRE) to simulate. This a good tool as it has inbuilt functions of LCD display, Shows all registers, ADC and DAC outputs, and many other features.

Instructions and their uses in Assembly 8051

- MOV: Used for moving data of 8 bit at a time, as 8051 is an 8bit architecture.

Ex: MOV destination, source;

- Arithmetic:

- ADD  Destination , Source

This adds the values in the source and destination registers, and stores the value in the Destination.

- ADDC :Add with carry

- BCD packed addition

- SUBB destination , source

Subtract with borrow, Subtracts source from destination along with borrow.

NOTE: to use it as SUB, make CY flag 0 in Status register.

- MUL AB: Multiplication of A and B values. Result lower byte in A and higher byte in B.

- DIV AB: Numerator in A, denominator in B. Result, quotient in A and remainder in B.

- Logical:

- ANL: And operation of dest, src.

- ORL: Or operation.

- XRL: Xor operation.

- RR source: right rotate source.

- RL source: left rotate source.

- RRC source: Right rotate carry.

- RLC source: Left Rotate carry.

- Jump and loop statements, these along with Subroutines and Interrupts can be further read in the Documentation of 8051.

8051 I/O Programming

- Rxd and Txd is from UART communication

- INT0 and INT1 are external interrupts.

- Pin 31 is connected to Vcc.

- XTAL 1&2 are for crystal oscillator +22pf ceramic capacitor.

- T0, T1, T2 are timer inputs for counter mode.

- MOSI (Master out slave in), MISO (Master in slave out) and SCK are used for Serial Peripheral Interface for ISP programming with the Arduino.

Assembly Programming

Program-1: Moving contents of a register

MOV SP, #1FH 	; Stack pointer is added with the address 1F

MOV R0, #25   	; move 25 to R0 register

MOV R1, #3CH	; move 3C to R1 register

MOV R2, #09H	; move 9 to R2 register

MOV B, #0DEH	; move DE to B register

Program-2: Multiplication as Repeated addition

MOV A, #0	; Initialize with 0

MOV R1, #10	; loop counter is 10

LOOP:   ADD A, #12	; adding 12 each iteration

DJNZ R1, LOOP	; decrement and jump if non zero

MOV R7, A	; move the answer in B to R7.

Program-3: Machine cycle and delay calculation

MACHINE CYCLE

DELAY: MOV R3, #19	; 1

HERE:  MOV R4, #255	; 1

LOOP: DJNZ R4, LOOP	; 2

DJNZ R3, HERE	; 2

RET	; 2

We are using 11.0596 MHz Crystal from AT89S52, hence one cycle will be 1.085 µs

LOOP takes (2x255) x 1.085 µs = 553.35 µs.

HERE repeats the LOOP 19 times, hence the total delay is

19x 553.35= 10513.65 µs, approximately 10ms.

These are approximates, not considering the subroutine instructions.

Program-4: Addition of values in register

MOV A, R0	; move values in R0 to A

ADD A, R1	; add values of R11 and values in A

JC TEXT		; if sum is greater than FFh then CY=1 (Carry Flag)

SJMP EXIT	; if CY=0 then exit

TEXT:  MOV A, #'Y'	; if CY=1 move 'Y' in to A

MOV P1, A	; send output via Port-1

EXIT:  NOP	; No Operation

END	; Terminate

Program-5: 50% duty cycle generation

BACK: CPL P1.0	; Complement the value in p1.0

LCALL DELAY	; call delay subroutine

SJMP BACK	; run in infinite loop

DELAY:

MOV R3, #19	; 1

HERE:  MOV R4, #255	; 1

LOOP:  DJNZ R4, LOOP	; 2

DJNZ R3, HERE	; 2

RET	; 2

Program-6: Read serial data and placed in output port

MOV R0, #08	; counter value 8

SETB P0.0	; make P0.0 as input

LOOP:    MOV C, P0.0	; move data from P0.0 to carry bit

RRC A	; right rotate with carry

DJNZ R0, LOOP	; repeat until all 8 bits are moved

MOV P1, A 	; transfer in a parallel format

C Programming Of 8051

Advantages:

- Platform Independent.

- Easier than Assembly.

- Many Libraries can be used.

Disadvantages:

- Limited memory in Microcontrollers, as C code takes more space than Assembly.

- High precision programming is better done in Assembly.

Data Type used in 8051 series:

The most preferred data type is ‘Unsigned Char’, as it occupies 8bit data. This is very much suitable for the 8bit architecture of 8051.

Avoid using other data types like ‘int’ as they occupy large memory in the microcontroller.

- Unsigned Char

- Unsigned Int

- Signed Char

- Signed Int

- Sbit

- Bit and sfr

Default the data type int or char will be signed, for unsigned we have to mention it explicitly.

Program Set-1

Program -1: To send Hex values of ASCII characters

#include <reg52.h>     					// reg51.h for 8051, reg52.h for AT89S52

void main (void)

{

unsigned char a[]= “0123456789ABCDEF”;

unsigned char z;

for(z=0;z<10;z++)

P1=a[z];

}

Program-2: Toggle LED delay version 1

#include<reg52.h>

void main(main)

{	unsigned int z;

while(1)			// Run Always, can also use for(;;)

{	P1=0x02;

for(z=0;z<50000;z++);	// Delay count

P1=0x00;

for(z=0;z<50000;z++);

}

}

Program-3: Toggle LED delay version 2

#include<reg52.h>

void delay(unsigned int x)

{	unsigned int i,j;

for(i=0;i<x;i++)

for(j=0;j<1275;j++);

}

void main(void)

{	while(1)

{	P2=0x01;

delay(70);

P2=0x00;

delay(70);

}

}

Program-4: Push button LED turn on

#include<reg52.h>

sbit x = P1^1;				//sbit to declare port

sbit led =P1^2;

void main(void)

{	x=1;     				//make x as input

while(1)			// to keep checking the status of the pin

{	if(x==0)			// active low input

led=1;

else

led=0;

}	}

Program-5: Switch or Sensor based alarm

#inlcude<reg52.h>

sbit in = P1^1;

sbit out = P1^2;

void delay(unsigned char z)

{	unsigned int m,n;

for(m=0;m<z;m++)

for(n=0;n<1275;n++);

}

void main(void)

{	while(in==0)

{	out=1;

delay(70);

out=0;

delay(70);

}	}

TIMER

Two timers Timer0 and Timer1 (in AT89S52 we have Timer2)

The timer register is divided into 2. The timer register is 16 bit long, but 8051 can access 8bit at a time, so the upper and lower byte are divided. Lower byte is TL0 & TL1 for timer0 and 1 respectively. Similarly higher byte TH0 & TH1.

Timer modes:

Mode 0: 13 bit timer / counter (8+5).

Mode 1: 16 bit timer / counter (max values FFFFh)

Mode 2: 8 bit timer/ counter	(max value FFh)

Mode 3: Split timer mode

C/T bit allows one to select whether the module should act as a timer or a counter. The 16 bit counter is very useful, if coupled along with 1Hz input, it can serve as a calendar for 136 years!!

Timer will use 1/ 12 th the frequency of the crystal oscillator, regardless of the machine cycle. To change the speed of the timer one must change the crystal.

Program Set -2

Program-1: LED blinking at exactly 1 second with timer module

#include<reg52.h>

void delay(void)

{	TMOD=0x10;

TL1=0xFE;

TH1=0xA5;        // this will give 500ms delay

TR1=1;

while(TF1==0);

TR1=0;

TF1=0;

}

void main(void)

{	unsigned int x;

while(1)

{

P0 =P0 ^ 0x01;

for(x=0;x<20;x++)

delay();

}

}

Program-2: Selection of frequency with button. Using timer for frequency generation

#include<reg52.h>

sbit led=P2^0; 			// led pin

sbit button=P1^2;		// switch pin

void delay(unsigned char y)

{	TMOD=0x10;

if(y==1)

{	TL1=0xFE;

TH1=0xA5;

}				// this will give 500ms delay

else

{	TL1=0xFE;

TH1=0x55;

}				//this will give 250ms delay

TR1=1;

while(TF1==0);

TR1=0;

TF1=0;

}

void main(void)

{	while(1)

{	led ^= 0x01;

if(button==0)

delay(0);

else

delay(1);

}	}

Program-3:

Using timer as a counter, to keep tracking the input pulse and display in output port. The             SIGNAL can be provided from Arduino generating a 1Hz square wave.

#include<reg52.h>

sbit freq=P3^5;		// For Timer1 INPUT PIN, can’t be replaced with any other (Pin 3.5)

void main(void)

{	freq=1;

TMOD=0x61;

TH1=0;

while(1)

{	do

{	TR1=1;

P1=TL1;

}

while(TF1==0);

TR1=0;

TF1=0;

}	}

Program-4: Generation of sine wave in C

#include<reg51.h>

void delay();

void main()

{	unsigned int i;

unsigned int wavelength[16]={ 127,176,218,245,255,245,218,176,128,79,27,10,0,10,37,79 };

while(1)

{	for(i=0;i<16;i++)

{	P1=wavelength[i];

}

}

}

MINI PROJECT

Development of a System that reads a 4 bit password, once the correct password is fed a countdown timer starts from 60. The timer runs for every 1 second, so countdown of 60 seconds precisely. The display is shown via Seven Segment with a decoder. After the countdown is completed, another task can be made to run. Here few LED glow in a certain pattern.	(Password is 2314)

Code

#include<reg52.h>

void password();	void wait();	void output();		void delay(int x);

sbit error=P2^7;	sbit pass0=P2^1;

sbit pass1=P2^2;	sbit pass2=P2^3;

sbit pass3=P2^4;	sbit out1=P1^7;

sbit out2=P2^6;		sbit out3=P3^6;		sbit out4=P3^7;

char seven[10]={0x00,0x02,0x04,0x06,0x08,0x0a,0x0c,0x0e,0x10,0x12};

void main()

{	int p=9,k=5;

error=0;

out1=0;		out2=0;

out3=0;		out4=0;

password();

P3=seven[6];

P1=seven[0];

delay(100);

P3=seven[5];

while(1)

{	for(p=9;p>=0;p--)

{	P1=seven[p];

delay(100);

}

k=k-1;

if(k==-1)

{	output();	}

P3=seven[k];

}

}

void password()

{	while(1)

{	 if(pass1==pass0==pass2==pass3==1)

{	wait();

if(pass2==0&&pass1==pass0==pass3==1)

{	wait();

if(pass1==0&&pass0==pass3==1)

{	wait();

if(pass3==0&&pass0==1)

{	wait();

if(pass0==0)

{ 	error=1;

return;

}

}

}

}

}

}

}

void wait()

{	int passwait0=pass0;	int passwait1=pass1;	int passwait2=pass2;	int passwait3=pass3;

while(passwait0==pass0&&passwait1==pass1&&passwait2==pass2&&passwait3==pass3)

{	}

}

void delay(int x)

{	if(x==0)

{	return;

}

TMOD=0x10;

TL1=0xff;

TH1=0xdb;

TR1=1;

while(TF1==0);

TR1=0;

TF1=0;

delay(x-1);

}

void output()

{	while(1)

{	out1=1;

delay(10);

out3=1;

delay(10);

out2=1;

delay(10);

out4=1;

delay(10);

out1=0;

delay(10);

out3=0;

delay(10);

out2=0;

delay(10);

out4=0;

delay(5);

out1=1;

out2=1;

out3=1;

out4=1;

delay(5);

out1=0;

out2=0;

out3=0;

out4=0;

delay(5);

}

}